[Appendix] Micrograph preprocessing script, mgprep.pl

#!/usr/bin/perl -w

# Preparing a set of micrograph frames for SPA

# Bernard Heymann

# 20160309 - 20190916

#

# usage: mgprep.pl in1.mrc [in2.mrc ...]

#

# The output will have file names that are derived from the input base names.

use strict;

# These parameters should be adapted for each specific case

my $bin = 2;			# Optional binning, set to 1 for no binning

my $px = 0.858;		# Pixel size before binning

my $dose = 1.0;		# Dose per fram in electrons/Å2

my $volt = 300;		# Acceleration voltage in kV

my $cs = 2.7;			# Spherical aberration in mm

my $amp = 0.07;		# Amplitude fraction

my $refnum = 8;		# Initial frame to choose as alignment reference

my $tile = "512,512,1";	# Tile size for power spectrum calculation

my $resaln = "20,1000";	# Limits for frame alignment

my $resctf = "5,20";	# Limits for CTF fitting

my $defrange = "0.5,3.5";	# Defocus range for CTF fitting

my $tilt = 0;			# Specimen tilt angle (set zero for no tilt)

my $axis = 0;			# Specimen tilt axis angle (ccw from x axis)

my $expos = 20;				# Total exposure time to calculate the drift rate

# Gain reference image, leave it empty if the micrographs are already gain-corrected

#my $gr = "../mg/gain_20160902.dm4";

my $gr = "";

if ( $gr ) { $gr = "-Gainreference " . $gr; }

my $pxb = $px * $bin;	# Pixel size after binning

foreach my $f (@ARGV) {

print "$f\n";

my $b = $f;

$b =~ s{\.[^.]+$}{};

my $cmd;

my $pf;

my $pfs = $b . "_sum.star";		# Frame alignment output parameter file

my $pfc = $b . "_ctf.star";		# CTF fit output parameter file

if ( ! -f $f ) {

print "Error: File not found! ($f)\n";

exit;

} elsif ( $f =~ /star/ ) {

$pf = $f;

} elsif ( $f =~ /[pif|tif|mrc|dm4]/ ) {

$pf = $b . ".star";

$cmd = "bmg -verb 7 -extract frame -Pixel $pxb -Volt $volt -Amp $amp -Cs $cs -out $pf $fb2";

run($cmd) if ! -f $pf;

} else {

print "Error: No useful filename specified! ($f)\n";

exit;

}

$cmd = "bseries -verb 1 -frames -dose $dose -align $refnum -resol $resaln -shift 10 -bin $bin -exposure $expos -write sum -out $pfs $pf";

run($cmd) if ! -f $pfs;

$cmd = "bctf -verb 1 -action prepfit -fitastig 1 -Range $defrange -basetype 4 -envtype 4 -tile $tile -sam $pxb -resol $resctf -tilt $tilt -axis $axis -Volt $volt -Amp $amp -Cs $cs -out $pfc $pfs";

run($cmd) if ! -f $pfc;

}

sub run {

my $cmd = shift @_;

print "\n$cmd\n";

open my $fh, "$cmd|" or die $!;

while ( <$fh> ) {

print;

}

close $fh;

}