Appendix

ÄKTAxpress Glycoprotein Purification Programme

(Main)

0.00 Base CV 5.027 {ml} HisTrap_FF_5_ml

0.00 Block Set_up_System_1

(Set_up_System_1)

0.00 Base SameAsMain

0.00 ResetFracPosition

0.00 PumpWash A1

0.00 SystemWash 10.00 {ml}

0.00 Alarm_Pressure Enabled 0.7 (Dhama et al.) 0.000 (Dhama et al.)

0.00 Flow 8 {ml/min} Buffer 0.10 {ml/min} Yes

0.00 ColumnPosition Position1

0.00 Set_Mark “Equilibration of His Column 1”

3.00 AutoZeroUV

3.00 ColumnPosition Bypass

3.00 Flow 0.00 {ml/min} Buffer 0.10 {ml/min} No

3.00 End_Block

0.00 Watch_AirSensor Equal 1 Finish_load_1 (Finish_load_1)

0.00 Base Volume

0.00 InletValve A1

0.00 Flow 0.00 {ml/min} Buffer 0.10 {ml/min} No 0.00 Block Air_removal_1

(Air_removal_1)

0.00 Base Volume

0.00 InletValve A1

0.00 Set_Mark “Air removal”

0.00 InjectionValve Waste

0.00 Flow 20 {ml/min} Buffer 0.10 {ml/min} Yes 20.00 End_Block

0.00 Block Wash_Column_1

(Wash_Column_1)

0.00 Base SameAsMain

0.00 Set_Mark “Washing column 1”

0.00 Flow 0.00 {ml/min} Buffer 0.10 {ml/min} No 0.00 Watch_Off AirSensor

0.00 Alarm_AirSensor Enabled

0.00 InjectionValve Inject

0.00 Flow 5.00 {ml/min} Buffer 0.10 {ml/min} Yes 10.00 AutoZeroUV

10.00 End_Block

0.00 Block End_fractionation_load_1 (End_fractionation_load_1)

0.00 Base SameAsMain

0.00 OutletValve WasteF1

0.00 End_Block

0.00 Block Set_up_elution_1

(Set_up_elution_1)

0.00 Base SameAsMain

0.00 ColumnPosition Bypass 0.00 PumpWash A3

0.00 OutletValve LoopFracF12 0.00 LoopSelection Bypass 0.00 ColumnPosition Position1 0.00 End_Block

0.00 Block Elution_Peak

(Elution_Peak)

0.00 Base SameAsMain 5.027 {ml} HisTrap_HP_5_ml

0.00 Watch_UV Greater_Than 100.000 (Wec et al.) Peak_Collection

(Peak_Collection)

0.00 Base Volume

0.00 LoopSelection LP1

0.00 Set_Mark “Peak_Start”

0.00 Watch_UV Less_Than 100.000 (Wec et al.) Peak_End

(Peak_End)

0.00 Base Volume

0.00 LoopSelection Bypass 0.00 Set_Mark “Peak_End” 0.00 End_Block

7.50 Block Peak_End

(Peak_End)

0.00 Base Volume

0.00 LoopSelection Bypass 0.00 Set_Mark “Peak_End” 0.00 End_Block

7.50 End_Block

5.00 Watch_Off UV

5.00 LoopSelection Bypass 5.00 End_Block

0.00 Block Set_up_Gel_Filtration

(Set_up_Gel_Filtration)

0.00 Base SameAsMain

0.00 Flow 0.00 {ml/min} Buffer 0.10 {ml/min} No 0.00 ColumnPosition Bypass

0.00 OutletValve WasteF1

0.00 PumpWash A4

0.00 SystemWash 10.00 {ml}

0.00 Alarm_Pressure Enabled 0.5 (Dhama et al.) 0.000 (Dhama et al.) 0.00 Alarm_AirSensor Enabled

0.00 Flow 1.2 {ml/min} Buffer 0.10 {ml/min} Yes 0.00 End_Block

0.00 Block Equilibrate_Gel_Filtration

(Equilibrate_Gel_Filtration)

0.00 Base CV 120.637 {ml} HiLoad_16/60_Superdex_75_prep_grade 0.00 ColumnPosition Position5

0.10 AutoZeroUV

0.10 End_Block

0.00 Block Gel_Filtration

(Gel_Filtration)

0.00 Base CV 120.637 {ml} HiLoad_16/60_Superdex_75_prep_grade 0.00 Block Inject_Peak

(Inject_Peak)

0.00 Base Volume

0.00 InjectionValve Reinject 0.00 LoopSelection LP1

0.00 Set_Mark “Peak_injection” 10.00 InjectionValve Inject 10.00 End_Block

0.30 Block Fractionation_Gel_Filtration

(Fractionation_Gel_Filtration)

0.00 Base CV 120.637 {ml} HiLoad_16/60_Superdex_75_prep_grade 0.00 OutletValve FracCollF2

0.00 Fractionation 2.000 {ml}

0.00 End_Block 1.10 End_Block 0.00 End_Method

0.00 Block Fractionation_load_1 (Fractionation_load_1)

0.00 Base SameAsMain

0.00 OutletValve F3

0.00 End_Block

0.00 Block Load_Sample_1

(Load_Sample_1)

0.00 Base Volume

0.00 Flow 8 {ml/min} Buffer 0.10 {ml/min} Yes 0.00 ColumnPosition Position1

0.00 Set_Mark “Loading sample 1”

0.00 Loop (200)#No_of_loops_for_load

0.00 InletValve A2 (50.00)#Volume_of_part_sample InletValve A1 60.00 Loop_End

60.00 End_Block

0.00 Message “The programme finished but there is still sample!!” Screen “error” 0.00 End_Method