Python code for visualizing and analyzing the growth curve data from microplate readers

Note: The code was written with Python 3.7, and tested for Python 3.7, 3.8, and 3.9.

# The OD and fluorescence data of the blank, sample S1, and sample S2 are stored in “blank_od.txt”, “blank_fl.txt”, “s1_fl.txt”, “s1_fl.txt”, “s2_od.txt”, “s2_fl.txt”, respectively. # %% import packages import matplotlib.pyplot as plt import matplotlib as mpl mpl.rcParams['font.size'] = 20 import numpy as np import pandas as pd from scipy.integrate import odeint from scipy.optimize import curve_fit from scipy.signal import find_peaks, peak_widths # %% define useful functions def ave(data): '''calculate the mean''' return data.mean(axis=1) def std(data): '''calculate the standard deviation''' return data.std(axis=1) def sem(data): '''calculate the error (standard error of the mean)''' return data.std(axis=1)/np.sqrt(data.shape[-1]) def calc_stats(sample_fn, blank_fn, dt=10.0/60.0, convfac=1): ''' calculate the statistics at all time points for the data stored in file [data_fn] ==== Parameters ==== * sample_fn: file name of the sample data file * blank_fn: file name of the blank data file * dt: time interval in hr (default: 10/60 hr = 10 min) * convfac: conversion factor for presenting the data (default: 1) ==== Returns ==== * t: time * sample_ave: mean values * sample_err: errors ''' blank = np.loadtxt(blank_fn) sample = np.loadtxt(sample_fn) if blank.shape[0] == sample.shape[0]: t = np.arange(0, blank.shape[0])*dt sample_ave = (ave(sample) - ave(blank))*convfac sample_err = np.sqrt(sem(sample)**2+sem(blank)**2) return (t, sample_ave, sample_err) else: print('Error: blank and sample do not have the same number of data/time points') return None def remove_baseline_from_data(sample_fn, blank_fn): ''' remove the baseline from sample data (baseline = average of blank data) ==== Parameters ==== * sample_fn: file name of the sample data file * blank_fn: file name of the blank data file ==== Returns ==== * new_sample_data: array ''' blank = np.loadtxt(blank_fn) sample = np.loadtxt(sample_fn) new_sample_data = np.zeros_like(sample) baseline = ave(blank) if blank.shape[0] == sample.shape[0]: for col in range(sample.shape[-1]): new_sample_data[:,col] = sample[:,col] - baseline return new_sample_data else: print('Error: blank and sample do not have the same number of data/time points') return None def smooth(x,window_len=11,window='hanning'): '''smooth the data (from online resources); credit/citation goes to https://scipy-cookbook.readthedocs.io/items/SignalSmooth.html ''' if window_len<3: return x ss=np.r_[x[window_len-1:0:-1],x,x[-2:-window_len-1:-1]] if window == 'flat': #moving average w=np.ones(window_len,'d') else: w=eval('np.'+window+'(window_len)') y=np.convolve(w/w.sum(),ss,mode='valid') return y def calc_deriv(t, ys, deriv_order=1, sw=11): '''calculate the derivative; note that ys is an array for the data from multiple wells (we need the raw data to get the errors) ==== Parameters ==== * t: time (1d array) * ys: list of single-well data (list of 1d array) * deriv_order: order of derivative (default: 1); use only 1 or 2. * sw: smooth window size (default: 11) ==== Returns ==== * t: time * dys: average of derivatives * err_dys: error of derivatives (standard error of the mean) ''' dt = t[1]-t[0] dys = [] for y in ys: if deriv_order == 1: dys.append(np.diff(smooth(y, sw))/dt) elif deriv_order==2: dys.append(np.diff(smooth(np.diff(smooth(y, sw))/dt, sw))/dt) else: print('Higher order derivatives are not supported') dys.append(np.diff(smooth(y, sw))/dt) dys = np.array(dys) return (t, dys.mean(axis=0)[:len(t)], dys.std(axis=0)[:len(t)]/np.sqrt(len(ys))) def get_peak_info(t, y): res = [] peaks, _ = find_peaks(y, height=np.max(y)/2.0) for p in peaks: res.append([t[p], y[p]]) return np.array(res) # %% Absorption of two example samples plt.figure(figsize=(5,4)) t1, s1, e1 = calc_stats('s1_od.txt', 'blank_od.txt') plt.errorbar(t1, s1, e1, fmt='o-',capsize=4, ms=12, mew=2, elinewidth=2, color='tab:blue', mfc='none', label='S1') t2, s2, e2 = calc_stats('s2_od.txt', 'blank_od.txt') plt.errorbar(t2, s2, e2, fmt='s-',capsize=4, ms=12, mew=2, elinewidth=2, color='tab:orange', mfc='none', label='S2') plt.xlabel('Time (hr)') plt.ylabel('A (a.u.)') plt.legend() plt.tight_layout() plt.savefig('./fig_abs_vs_t.pdf') # %% Fluorescence of two example samples plt.figure(figsize=(5,4)) scfac = 1e5 t1, s1, e1 = calc_stats('s1_fl.txt', 'blank_fl.txt') plt.errorbar(t1, s1/scfac, e1/scfac, fmt='o-',ms=12, mew=2, capsize=4, elinewidth=2, color='tab:blue', mfc='none',label='S1') t2, s2, e2 = calc_stats('s2_fl.txt', 'blank_fl.txt') plt.errorbar(t2, s2/scfac, e2/scfac, fmt='s-',ms=12, mew=2, capsize=4, elinewidth=2, color='tab:orange', mfc='none',label='S2') plt.xlabel('Time (hr)') plt.ylabel(r'F ($\times 10^5$ a.u.)') plt.legend() plt.tight_layout() plt.savefig('./fig_fl_vs_t.pdf') # %% derivative of absorption of two example samples t,s,e = calc_stats('s1_od.txt', 'blank_od.txt') sample = remove_baseline_from_data('s1_od.txt', 'blank_od.txt') ys = [sample[:,col] for col in range(sample.shape[-1])] t1, d1, e1 = calc_deriv(t, ys, deriv_order=1, sw=11) p1 = get_peak_info(t1, d1)[0] t,s,e = calc_stats('s2_od.txt', 'blank_od.txt') sample = remove_baseline_from_data('s2_od.txt', 'blank_od.txt') ys = [sample[:,col] for col in range(sample.shape[-1])] t2, d2, e2 = calc_deriv(t, ys, deriv_order=1, sw=11) p2 = get_peak_info(t2, d2)[0] scfac = 1 plt.figure(figsize=(5,4)) plt.errorbar(t1, d1/scfac, e1/scfac, fmt='o-',ms=12, mew=2, capsize=4, elinewidth=2, color='tab:blue', mfc='none',label='S1') plt.errorbar(t2, d2/scfac, e2/scfac, fmt='s-',ms=12, mew=2, capsize=4, elinewidth=2, color='tab:orange', mfc='none',label='S2') plt.xlabel('Time (hr)') plt.ylabel(r'dA/dt (a.u.)') ylim = plt.ylim() plt.plot([p1[0],p1[0]],ylim, '--', color='black', zorder=99) plt.plot([p2[0],p2[0]],ylim, '--', color='black', zorder=99) print(p1, p2, p2-p1) plt.ylim(ylim) # plt.legend() plt.tight_layout() plt.savefig('./fig_dA_vs_t.pdf') # %% derivative of fluorescence of two example samples t,s,e = calc_stats('s1_fl.txt', 'blank_fl.txt') sample = remove_baseline_from_data('s1_fl.txt', 'blank_fl.txt') ys = [sample[:,col] for col in range(sample.shape[-1])] t1, d1, e1 = calc_deriv(t, ys, deriv_order=2, sw=11) p1 = get_peak_info(t1, d1)[0] t,s,e = calc_stats('s2_fl.txt', 'blank_fl.txt') sample = remove_baseline_from_data('s2_fl.txt', 'blank_fl.txt') ys = [sample[:,col] for col in range(sample.shape[-1])] t2, d2, e2 = calc_deriv(t, ys, deriv_order=2, sw=11) p2 = get_peak_info(t2, d2)[0] scfac = 1e5 plt.figure(figsize=(5,4)) plt.errorbar(t1, d1/scfac, e1/scfac, fmt='o-',ms=12, mew=2, capsize=4, elinewidth=2, color='tab:blue', mfc='none',label='S1') plt.errorbar(t2, d2/scfac, e2/scfac, fmt='s-',ms=12, mew=2, capsize=4, elinewidth=2, color='tab:orange', mfc='none',label='S2') plt.xlabel('Time (hr)') plt.ylabel(r'd$^2$F/dt$^2$ ($\times 10^5$ a.u.)') ylim = plt.ylim() plt.plot([p1[0],p1[0]],ylim, '--', color='black', zorder=99) plt.plot([p2[0],p2[0]],ylim, '--', color='black', zorder=99) plt.ylim(ylim) print(p1, p2, p2-p1) # plt.legend() plt.tight_layout() plt.savefig('./fig_dF_vs_t.pdf')