/*

P1-Rotate_Resize_Normalize

Part I of Side-averaging and analysis (three parts in total)

Function:  To rotate, resize and normalize side-view images for the subsequent side-averaging. It prepares images for averaging, but it does not average the images. To average images, you need to use ImageJ>Image>Stacks>Z-project.

Input: The side-view multi-color image stack (one channel must be the giantin double-punctum image) must meet the below requirements. (1) The background is subtracted to zer. (2) The image is a square (40 x 40 pixels). (3) The trans-side of the Golgi is up. (4) Two giantin puncta are manually marked as two ROIs and added to the ROI Manager.

Usage: Open the multi-color image stack. Select the giantin channel as the active window. Execute this macro. A window pops up to prompt the selection of the folder directory where processed images are saved.

Each channel image is individually saved with "C" and the channel number appended at the beginning of its file name. In addition, relevant information, file names, and folder paths of processed images are displayed in the "Log" window.

To acquire the final side-averaged image, the resulting images of the test protein are stacked and subjected to Z-projection averaging (Image>Stacks>Z Project: Average Intensity).

*/

macro "P1-Rotation_Resize_Normalize"

{

folder_path=getDirectory("Choose a Directory")

run("Clear Results");

title=getTitle;

selectWindow(title);

//This line of code is added to change the property of the image so that the dimension unit is pixel instead of the micrometer.

run("Properties...", "channels=3 slices=1 frames=1 unit=pixel pixel_width=1 pixel_height=1 voxel_depth=1");

//Two ROIs are selected for the two Giantin puncta; The two ROIs are included in the ROI Manager as the first and second item.

//Results window pops up after the measurement.

run("Set Measurements...", "center redirect=None decimal=4");

roiManager("Deselect");

roiManager("Measure");

//(x1,y1) is the coordinate of the left Giantin punctate's center of fluorescence mass

//(x2,y2) is the coordinate of the right Giantin punctate's center of fluorescence mass

x1=getResult("XM", 0);

y1=getResult("YM", 0);

x2=getResult("XM", 1);

y2=getResult("YM", 1);

print("x1= ", x1, "; y1= ", y1, "; x2= ", x2, "; y2= ", y2);

//(xc, yc) is the coordinate of the center of the mass of Giantin double-puncta

run("Clear Results");

run("Set Measurements...", "center redirect=None decimal=4");

run("Select All");

run("Measure");

xc=getResult("XM", 0);

yc=getResult("YM", 0);

print("xc= ", xc, "; yc= ", yc);

//translate Giantin center, (xc,yc), to the image center

Image_h=getHeight();

x_shift=(Image_h/2)-xc;

y_shift=(Image_h/2)-yc;

run("Translate...", "x=&x_shift y=&y_shift interpolation=Bilinear");

print("x_shift= ", x_shift, "; y_shift= ", y_shift);

//rotate the image around the center so that Giantin puncta are horizontal

Angle=0-(57.2958*atan((y2-yc)/(x2-xc)));

run("Rotate... ", "angle=&Angle grid=2 interpolation=Bilinear");

print("Rotation angle (degree) = ", Angle);

//d is the radius of the Giantin doulbe-puncta (ring), which is the distance fromt (xc, yc) to either (x1, y1) or (x2, y2).

//Note: it is not the lateral size of Giantin.

d=(sqrt((x2-x1)*(x2-x1)+(y2-y1)*(y2-y1)))/2;

print("The radius of the Giantin puncta = ", d);

//below is the module to resize the width of the image to normalize the radius of the Giantin puncta d to 80 pixels

//the height of the image is expanded 8 times;

selectWindow(title);

Image_wh=getHeight();

New_w=Image_wh*80/d;

New_h=Image_wh*8;

//print("Original width of the image= ", Image_wh, "; Expanded width of the new image= ", New_w, "; Expanded height of the enew image= ", New_h);

run("Size...", "width=&New_w height=&New_h interpolation=Bilinear");

//below is the module to resize the canvas to a 701x701 square.

run("Canvas Size...", "width=701 height=701 position=Center zero");

//print("Canvas size is further expanded to 701x701 pixel.");

//below is the module to normalize the integrated intensity to 50 million by using Process->Math for each channel

getDimensions(width1, height1, channels, slice1, frame1);

for (channel_i=1; channel_i<=channels; channel_i++)

{

Stack.setChannel(channel_i);

run("Set Measurements...", "integrated redirect=None decimal=4");

run("Select All");

run("Measure");

Integrated_intensity=getResult("IntDen", nResults-1);

I_factor=50000000/Integrated_intensity;

run("Multiply...", "value=&I_factor slice");

//print("channel=", channel_i," done.");

}

//print("Channel integrated intensity normalized to 50,000,000.");

//below is the module to split channels and save each channel image to a specified path.

getDimensions(width1, height1, channels, slice1, frame1);

run("Split Channels");

for (channel_i=1; channel_i<=channels; channel_i++)

{selectWindow("C"+channel_i+"-"+title);

saveAs("Tiff", folder_path+"C"+channel_i+"-"+title);  //string concatenation

}

print("The image file:  ", title);

print("Processed files are saved at= ", folder_path);

print("--------------------------------------------------------------------------------------------------------------------------");

roiManager("Delete");

}