/*

P2-Resize_Add_Line

Part II of side-averaging and analysis (three parts in total)

Function: 1) It resizes the image's width and height to 328*701 pixels so that it is isotropic in scale (5.6 nm/pixel). 2) It adds a horizontal white line to indicate the center of the fluorescence mass (ym).

Input: Side-averaged image (tif) produced by Part I (P1-Rotate_Resize_Normalize). The input image must be 701*701 pixels.

Output: Two images are generated: 1) resized1 image and 2) resized2. The resized2 image has the overlay of a white horizontal line across its center of the fluorescence mass (ym).

Usage: Open input images in ImageJ or Fiji and execute this macro. A window pops up to prompt the selection of the folder directory where the processed images are to be saved.

Two images are generated at the specified folder: 1) resized image with "resized1" appended at the beginning of the file name, and 2) a white line overlayed resized image with "resized2" appended at the beginning of the file name.

File names and folder paths of processed images are displayed in the "Log" window.

*/

//Created on 23 December 2020.

//Modified on 07 January 2021. The image size was corrected.

//Comments were edited on 5 June 2021

macro "P2-Resize_Add_Line"

{

//Select the folder to save the processed images.

folder_path=getDirectory("Choose a Directory");

run("Clear Results");

title=getTitle;

selectWindow(title);

//Check if the input image is 701*701 pixel

if (getHeight()!=701 || getWidth()!=701)

{print ("ERROR. The size of the image should be 701x701 pixel!");

exit;

}

//Resize the image to width = 328 pixel; height = 701 pixel;

New_w=328;

New_h=701;

run("Size...", "width=&New_w height=&New_h interpolation=Bilinear");

//Save the resized image by adding "resized1" at the beginning of the file name.

saveAs("Tiff", folder_path+"resized1-"+title);  //string concatenation

//Print the file name.

print("The resized image file:  ", "resized1-"+title);

//Acquire the center of mass of the image.

run("Set Measurements...", "center redirect=None decimal=4");

run("Select All");

run("Measure");

//Results window pops up and send the column "YM" value to y1; x1 is not useful here.

x1=getResult("XM", 0);

y1=getResult("YM", 0);

print("Center of mass: x1= ", x1, "; y1= ", y1);

//Generate an image with the overlay of a horizontal line indicating the center of mass.

getDimensions(width1, height1, channels, slice1, frame1);

setColor(65535);

setLineWidth(1);

drawLine(0, y1, width1-1, y1);

//Save the resized and line-overlayed image by adding "resized2-" at the beginning of the file name.

saveAs("Tiff", folder_path+"resized2-"+title);  //string concatenation

//Print the file name and directory.

print("The resized and line-overlayed image file:  ", "resized2-"+title);

print("Both processed files are saved at= ", folder_path);

print("--------------------------------------------------------------------------------------------------------------------------");

}